SOFTWARE VERIFICATION PLAN

SVP

Intelligent Safety-Critical Engineering Environment Toolchain

DO-178C Lifecycle Automation (M1 → M6)

| Field | Value |
| --- | --- |
| Document ID | SVP-TOOLCHAIN-001 |
| Version | 2.0 |
| Status | DRAFT |
| Date | 2026-06-18 |
| Author | Hatem ADOUANI |
| Project | I-SCEET |
| Institution | Capgemini Engineering / ESPRIT |
| Supervisor | [Supervisor Name] |
| DAL Level | A (Assumed for demonstration purposes in an Engine Control-like safety-critical context) |
| Standard | RTCA DO-178C / EUROCAE ED-12C + DO-330 |

# 1. Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-04-19 | Hatem ADOUANI | Initial draft — project kickoff |
| 2.0 | 2026-06-18 | Hatem ADOUANI | Added AI model independence verification (Sec. 3.3): M4 (Gemma3 27B) independent of M3 (Qwen2.5-Coder 32B); M5 (DeepSeek-R1 32B) independent of M1 (Qwen3 32B) |

# 2. Introduction

## 2.1 Purpose

This Software Verification Plan (SVP) defines the verification processes, methods, activities, and responsibilities for the Intelligent Safety-Critical Engineering Environment Toolchain (I-SCEET) project. The SVP establishes how each AI-generated lifecycle artifact will be verified against DO-178C objectives prior to use in downstream activities.

This document shall be reviewed and approved prior to the commencement of any verification activity and is a required deliverable for SOI #1.

## 2.2 Project Overview

The I-SCEET project is an AI-powered toolchain composed of six intelligent modules (M1 to M6). Verification activities apply to the outputs of each module as follows:

| Module | Name | Input | Output Document | Verification Document | DO-178C § |
| --- | --- | --- | --- | --- | --- |
| M1 | HLR Generator | PDS + HW Doc + System Architecture | SRD-001 | Review record + SRD-001 sign-off | §5 |
| M2 | LLR Generator | SRD-001 (HLRs) | SDDD-001 | Review record + SDDD-001 sign-off | §6 |
| M3 | Code Generator | SDDD-001 (LLRs) | /src folder | Code review + static analysis report | §7 |
| M4 | LLT Generator | SDDD-001 (LLRs) | SVCP-LLT-001 | LLT review record + coverage report | §9 |
| M5 | HLT Generator | SRD-001 (HLRs) | SVCP-HLT-001 | HLT review record + coverage report | §9 |
| M6 | Traceability Generator | All artifacts | STD-001 | Traceability audit record | §11 |

## 2.3 Scope

This SVP covers all verification activities performed on AI-generated artifacts within the I-SCEET project. It applies to reviews, analyses, testing, coverage measurement, and traceability verification. It does not cover the development activities themselves, which are governed by SDP-TOOLCHAIN-001.

## 2.4 Referenced Documents

- RTCA DO-178C — Software Considerations in Airborne Systems and Equipment Certification (2012)

- RTCA DO-330 — Software Tool Qualification Considerations

- SAE ARP4754A — Guidelines for Development of Civil Aircraft and Systems

- SAE ARP4761 — Guidelines for Conducting Safety Assessments

- PSAC-TOOLCHAIN-001 — Plan for Software Aspects of Certification

- SDP-TOOLCHAIN-001 — Software Development Plan

- SCMP-TOOLCHAIN-001 — Software Configuration Management Plan

- SQAP-TOOLCHAIN-001 — Software Quality Assurance Plan

- SRS-001 — Software Requirements Standards

- SDS-001 — Software Design Standards

- SCS-001 — Software Code Standards

# 3. Verification Overview

## 3.1 Verification Objectives

The verification process shall demonstrate that the software lifecycle data produced by each AI module satisfies the applicable DO-178C objectives. In accordance with DO-178C §6.3, verification shall establish that:

- Requirements are accurate, consistent, verifiable, and traceable

- The software design correctly and completely implements the requirements

- The source code correctly implements the design

- Test cases adequately cover all requirements and achieve required structural coverage

- Traceability is complete and bidirectional across all lifecycle artifacts

## 3.2 Verification Approach

Each AI-generated artifact follows a generate-then-verify model. No artifact may be used as input to a subsequent module until it has been independently reviewed and all action items closed. The following verification methods are applied:

| Method | Description | Applied to |
| --- | --- | --- |
| Review / Inspection | Independent engineer reads the artifact against a DO-178C checklist and raises action items for any deficiency | SRD-001, SDDD-001, /src, SVCP-LLT-001, SVCP-HLT-001, STD-001 |
| Analysis | Formal assessment of properties such as traceability completeness, complexity, MISRA-C compliance, orphan detection | LLRs, Source Code, Traceability Matrix |
| Testing | Execution of generated test cases (SVCP-LLT-001, SVCP-HLT-001) against source code and measurement of structural coverage | /src folder |
| Coverage Analysis | Measurement of statement, decision, and MC/DC coverage against DAL A objectives using a qualified coverage tool | /src folder — all modules |

## 3.3 Verification Independence

Per DO-178C §6.3.2, verification activities shall be performed by personnel independent from those who produced the artifact under review. Independence is defined as:

- The reviewer has not configured, prompted, or tuned the AI module that produced the artifact

- The reviewer has not authored the input document(s) fed to the AI module

- At the AI module level, independent verification modules (M4, M5) shall use a distinct model and prompt from the generation modules they verify: M4 (Gemma3 27B) is independent of M3 (Qwen2.5-Coder 32B); M5 (DeepSeek-R1-Distill 32B) is independent of M1 (Qwen3 32B), per SCMP-TOOLCHAIN-001 §3.3

- For DAL A, tool-generated verification outputs (coverage reports, static analysis) shall additionally be reviewed by an independent engineer

# 4. Verification Activities by Module

## 4.1 M1 Output Verification — SRD-001 (HLRs)

The HLRs generated by M1 and the System Architecture section provided by the engineer shall be verified as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| HLR Review | Each HLR conforms to SRS-001 format rules (ID, 'shall' language, no ambiguity) | §6.3.1a — Accuracy | Independent Engineer |
| Completeness Check | Every PDS item has at least one HLR child; no unaddressed system requirement | §6.3.1b — Completeness | Independent Engineer |
| Consistency Check | No contradictions exist between HLRs | §6.3.1c — Consistency | Independent Engineer |
| Verifiability Check | Each HLR can be verified by inspection, analysis, or test | §6.3.1d — Verifiability | Independent Engineer |
| Derived Req. Review | All [DERIVED] HLRs identified, justified, and submitted to safety process | §6.3.1e — Traceability | Systems Engineer + QA |
| Traceability Check | Each HLR is linked to at least one PDS item; STD-001 updated | §6.3.1f — Traceability | Independent Engineer |
| System Architecture Review | Architecture section is consistent with HW doc and PDS inputs | §6.3.2 — Accuracy | Engineer |

## 4.2 M2 Output Verification — SDDD-001 (Software Architecture + LLRs)

The Software Architecture and LLRs generated by M2 shall be verified as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| LLR Review | Each LLR conforms to SRS-001 format; correct 'shall' language; no ambiguity | §6.3.2a — Accuracy | Independent Engineer |
| HLR-to-LLR Completeness | Each HLR is fully addressed by at least one LLR; no HLR left unimplemented | §6.3.2b — Completeness | Independent Engineer |
| No Undocumented Functions | No LLR introduces functionality not traceable to an HLR (unless [DERIVED]) | §6.3.2c — Consistency | Independent Engineer |
| Architecture Review | Module decomposition, interfaces, and data flows correctly implement HLRs | §6.3.2d — Design accuracy | Independent Engineer |
| Complexity Analysis | No module exceeds complexity limits defined in SDS-001 | §6.3.2e — Standards | Independent Engineer |
| Traceability Update | LLR-to-HLR links established and recorded in STD-001 | §6.3.2f — Traceability | Independent Engineer |

## 4.3 M3 Output Verification — /src Folder (Source Code)

The source code generated by M3 shall be verified as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| Code Review | Each source file reviewed against SCS-001 checklist: naming, commenting, structure | §6.3.3a — Accuracy | Independent Engineer |
| LLR-to-Code Traceability | Each LLR is implemented by at least one function; header comments reference LLR IDs | §6.3.3b — Completeness | Independent Engineer |
| MISRA-C Analysis | Static analysis tool run on all /src files; all mandatory rules satisfied; deviations logged | §6.3.3c — Standards | Independent Engineer |
| No Dead Code | No unreachable code paths exist in any /src file | §6.3.3d — Consistency | Independent Engineer |
| Stack / Memory Usage | No dynamic memory allocation; stack depth estimated and within hardware limits | §6.3.3e — Safety | Independent Engineer |
| Code-to-LLR Traceability | All code traced to at least one LLR; orphan code reported to M6 | §6.3.3f — Traceability | Independent Engineer |

## 4.4 M4 Output Verification — SVCP-LLT-001 (Low Level Tests)

M4 reads EXCLUSIVELY from SDDD-001 (LLRs). The generated LLT cases shall be verified as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| LLR Coverage Review | Each LLR has at least one normal-range test case and one robustness test case | §6.4.2.1a — Coverage | Independent Engineer |
| Test Case Accuracy | Expected results defined for each test case; no ambiguous pass/fail criteria | §6.4.2.1b — Accuracy | Independent Engineer |
| Structural Coverage (DAL A) | Statement + Decision + MC/DC coverage achieved on all /src files | §6.4.4.2 — MC/DC | Coverage Tool + Engineer |
| Input Boundary Analysis | Boundary values, equivalence classes, and invalid inputs covered per LLR | §6.4.2.1c — Robustness | Independent Engineer |
| LLT-to-LLR Traceability | Each test case traces to at least one LLR; no orphan test cases | §6.4.2.2 — Traceability | Independent Engineer |
| Test Independence | LLT author has not configured M4 or authored SDDD-001 LLRs | §6.3.2 — Independence | QA |

## 4.5 M5 Output Verification — SVCP-HLT-001 (High Level Tests)

M5 reads EXCLUSIVELY from SRD-001 (HLRs). The generated HLT cases shall be verified as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| HLR Coverage Review | Each HLR has at least one normal-range test case validating functional behavior | §6.4.1.1a — Coverage | Independent Engineer |
| Test Case Accuracy | Expected results defined for each test case; no ambiguous pass/fail criteria | §6.4.1.1b — Accuracy | Independent Engineer |
| Boundary & Robustness | Performance and timing requirements verified; out-of-range inputs tested per HLR | §6.4.1.1c — Robustness | Independent Engineer |
| HLT-to-HLR Traceability | Each test case traces to at least one HLR in SRD-001; no orphan test cases | §6.4.1.2 — Traceability | Independent Engineer |
| No LLR Leakage | HLT cases reference only HLRs from SRD-001 — never LLRs from SDDD-001 | §6.4.1 — Independence | QA |
| Functional Independence | HLT author has not configured M5 or authored SRD-001 HLRs | §6.3.2 — Independence | QA |

## 4.6 M6 Output Verification — STD-001 (Traceability Matrix)

The traceability matrix generated by M6 shall be audited as follows:

| Activity | Criterion | DO-178C Objective | Performed By |
| --- | --- | --- | --- |
| Upward Traceability | Each HLR traces to at least one PDS item; each LLR traces to at least one HLR | §6.5a — Completeness | Independent Engineer |
| Downward Traceability | Each PDS item covered by HLR; each HLR covered by LLR; each LLR covered by code | §6.5b — Completeness | Independent Engineer |
| Test Traceability | Each HLR covered by at least one HLT; each LLR covered by at least one LLT | §6.5c — Coverage | Independent Engineer |
| Orphan Detection | No HLR, LLR, source file, or test case exists without a parent link in STD-001 | §6.5d — Completeness | QA + Engineer |
| Coverage Traceability | Coverage results (MC/DC) linked to corresponding LLR and test case IDs in STD-001 | §6.5e — Coverage | Independent Engineer |

# 5. Coverage Objectives

## 5.1 Structural Coverage by DAL

As this project assumes DAL A (Engine Control context), ALL three coverage levels are required on all /src files:

| DAL | Statement | Decision | MC/DC | Status for I-SCEET |
| --- | --- | --- | --- | --- |
| A | Required | Required | Required | ✓ APPLIES — all /src files |
| B | Required | Required | Not Required | N/A |
| C | Required | Not Required | Not Required | N/A |
| D | Not Required | Not Required | Not Required | N/A |

## 5.2 MC/DC Coverage Requirements (DAL A)

Modified Condition/Decision Coverage (MC/DC) requires that for every Boolean decision:

- Each condition in the decision has been shown to independently affect the outcome

- Each condition has been shown to take both a TRUE and FALSE value

- Each decision has been shown to take both a TRUE and FALSE outcome

M4-generated test cases (SVCP-LLT-001) must achieve full MC/DC on all /src files. Coverage gaps identified during execution must be reported as problem reports and resolved before SOI #3.

## 5.3 Coverage Tool

A qualified coverage analysis tool shall be used to measure structural coverage. The tool shall be qualified per DO-330 at the appropriate TQL level. Coverage results shall be recorded in the Software Verification Results (SVR-001) and linked to STD-001.

# 6. Verification Environment

## 6.1 Verification Tools

| Tool | Purpose | Qualification Required |
| --- | --- | --- |
| M4 — AI LLT Agent | Generate low level test cases from SDDD-001 LLRs | Yes — DO-330 TQL-4 |
| M5 — AI HLT Agent | Generate high level test cases from SRD-001 HLRs | Yes — DO-330 TQL-4 |
| M6 — Traceability Agent | Generate and audit STD-001 traceability matrix | Yes — DO-330 TQL-5 |
| Static Analysis Tool | MISRA-C compliance check on /src folder | Yes — DO-330 TQL-5 |
| Coverage Analysis Tool | Statement / Decision / MC/DC coverage on /src | Yes — DO-330 TQL-4 |
| [Test Execution Tool] | Execute SVCP-LLT-001 and SVCP-HLT-001 on target or host | TBD |
| [Version Control — Git] | Baseline verification artifacts | No |

## 6.2 Target Environment

Verification activities shall be performed on the following environment:

- Host OS: Linux (Debian 12) or Windows 11 — version controlled

- Target Hardware: STM32F303-based embedded system (for integration testing)

- Compiler: [TBD — version locked and configuration-managed]

- All tool versions and configurations shall be recorded in the Software Configuration Index (SCI)

# 7. Verification Results and Problem Reports

## 7.1 Software Verification Results (SVR-001)

All verification activities shall produce recorded results stored in SVR-001. For each verification activity, SVR-001 shall capture:

- Artifact verified (document ID, version, date)

- Verification method applied (review / analysis / test / coverage)

- Pass / Fail status for each criterion

- Reviewer name and independence confirmation

- Reference to any problem reports raised

- Final sign-off and date

## 7.2 Problem Reports

Any deficiency identified during a verification activity shall be recorded as a Problem Report (PR) in the project tracking system. Problem reports shall:

- Be assigned a unique ID (PR-NNN)

- Classify severity: Critical / Major / Minor

- Reference the artifact and criterion that failed

- Be resolved and closed before the artifact is accepted as verified

- Be tracked through to SOI gate closure per SCMP-TOOLCHAIN-001

## 7.3 Re-Verification Rules

If a verified artifact is modified after acceptance (e.g., due to a change request), the following re-verification rules apply:

- Any change to SRD-001 HLRs triggers re-verification of SDDD-001, /src, SVCP-HLT-001, and STD-001

- Any change to SDDD-001 LLRs triggers re-verification of /src, SVCP-LLT-001, and STD-001

- Any change to /src triggers re-execution of SVCP-LLT-001 and re-measurement of coverage

- Impact analysis must be performed and documented before re-verification scope is agreed

# 8. Transition Criteria

| Phase Transition | Entry Criteria | Exit Criteria | Responsible |
| --- | --- | --- | --- |
| SOI #1 → M1 | PSAC, SDP, SVP, SCMP, SQAP, SRS-001, SDS-001, SCS-001 all approved | Certification authority SOI #1 review passed; all planning docs baselined | Author + Supervisor + QA |
| M1 → M2 | SRD-001 produced by M1 | SRD-001 §2 (HLRs) reviewed; all PRs closed; SRD-001 baselined | Independent Engineer |
| M2 → M3 | SDDD-001 produced by M2; SRD-001 verified | SDDD-001 reviewed (architecture + LLRs); no undocumented functions; SDDD-001 baselined | Independent Engineer |
| M3 → M4/M5 | SDDD-001 verified; /src produced by M3 | /src reviewed; MISRA-C clean; LLR-to-code traceability complete; /src baselined | Independent Engineer |
| M4/M5 → M6 | SVCP-LLT-001 and SVCP-HLT-001 produced | Both test sets reviewed; coverage executed; MC/DC achieved; SVR-001 updated | Independent Engineer |
| M6 → SOI #3 | STD-001 produced by M6; all prior verifications complete | STD-001 audited; no orphan requirements or code; STD-001 baselined | QA + Engineer |
| SOI #3 → SOI #4 | All lifecycle data complete and baselined | SAS-001 compiled; certification authority SOI #4 review passed | Author + QA |

# 9. SOI Review Plan

| SOI | Timing | Verification Items Reviewed | Acceptance Criteria |
| --- | --- | --- | --- |
| SOI #1 | Before M1 starts | SVP-TOOLCHAIN-001 (this document), PSAC, SDP, SCMP, SQAP, SRS-001, SDS-001, SCS-001 | All planning and standards documents approved; verification approach accepted by certification authority |
| SOI #2 | After M1 + M2 + M3 | SRD-001 review record, SDDD-001 review record, /src review record, MISRA-C report | All artifacts reviewed, PRs closed, baselines established; no open critical or major issues |
| SOI #3 | After M4 + M5 + M6 | SVCP-LLT-001 review, SVCP-HLT-001 review, SVR-001 (coverage results), STD-001 audit record | MC/DC coverage achieved on all /src; traceability complete; no orphans; all PRs resolved |
| SOI #4 | Project end | SAS-001, full lifecycle data package | All DO-178C objectives for DAL A satisfied; SAS-001 signed off |

# 10. Configuration Management and Quality Assurance

## 10.1 Configuration Management

All verification artifacts (review records, SVR-001, PRs, coverage reports) shall be placed under configuration management per SCMP-TOOLCHAIN-001. Key rules:

- Verification records shall be baselined at each SOI gate alongside the artifacts they verify

- Any re-verification triggered by a change shall reference the original PR and be linked to the updated artifact baseline

- SVR-001 shall be version-controlled and updated with each new verification cycle

## 10.2 Quality Assurance

SQA verification activities are governed by SQAP-TOOLCHAIN-001. Verification-phase SQA activities include:

- Confirmation that no artifact was accepted as verified without a completed review record

- Audit that M4 only reads SDDD-001 and M5 only reads SRD-001 — mixing is a non-conformance

- Confirmation that coverage results meet DAL A MC/DC objectives before SOI #3

- Review of all open PRs before each SOI gate

# 11. Additional Considerations

## 11.1 Verification of Derived Requirements

HLRs tagged [DERIVED] in SRD-001 shall undergo an additional verification step: safety impact assessment per ARP4761. Derived LLRs in SDDD-001 shall similarly be flagged and reviewed. No derived requirement shall propagate to M3 or M4/M5 without explicit engineer approval.

## 11.2 AI Non-Determinism

AI modules (M4, M5) may produce different test cases across runs for identical inputs. To mitigate this:

- Model version and generation parameters (temperature, seed) shall be locked and configuration-managed

- If re-generation is required, all previously accepted test cases shall be re-reviewed

- Differences between generations shall be documented as a change request

## 11.3 Coverage Gaps

If structural coverage analysis reveals uncovered code paths after M4 test execution, the following process applies:

- A problem report (PR) shall be raised identifying the uncovered path and its source LLR

- M4 may be re-run with additional constraints to generate supplementary test cases

- If coverage cannot be achieved, a formal justification (dead code analysis) shall be produced and accepted by QA

## 11.4 Limitations of AI-Generated Test Cases

- AI-generated test cases may not cover all corner cases — mitigated by mandatory independent review against DO-178C §9 objectives

- AI-generated expected results may contain errors — mitigated by engineer validation of each expected result prior to test execution

- Coverage tool results are only valid if the tool is qualified per DO-330 — tool qualification must be confirmed before SOI #3

# 12. Approvals

This document requires the following approvals before SOI #1:

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Author | Hatem ADOUANI |  |  |
| Project Supervisor | [Supervisor Name] |  |  |
| Quality Assurance | [QA Name] |  |  |
| Certification Authority | [Authority Rep] |  |  |