SOFTWARE REQUIREMENTS DOCUMENT

Intelligent Safety-Critical Engineering Environment Toolchain

| Document ID | SRD-001 |
| --- | --- |
| Version | 1.4 |
| Status | DRAFT |
| Date | 2026-07-11 |
| Author | M1 (I-SCEET — Qwen3 32B) |
| Project | stm32f303_boot_drv |
| Institution | Personal project |
| DAL Level | A |
| Standard | RTCA DO-178C §5 |

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.4 | 2026-07-11 | M1 (I-SCEET AI) | AI-generated document |

SECTION I — INTRODUCTION

I.1 Purpose

The Software Requirements Document (SRD) defines the high-level functional and non-functional requirements for the I-SCEET STM32F303VCT6 bootloader, driver layer, and minimal application. It serves as the foundation for M2 (SDDD/LLR generation) and downstream DO-178C DAL A certification activities, ensuring traceability from PDS to HLR to LLR to code.

I.2 Scope

This SRD applies to the I-SCEET project targeting STM32F303VCT6 microcontroller. It covers:

- Bootloader initialization sequence (RCC, SysTick, GPIO, UART, Flash, CRC, WDG)

- Clock configuration to 72 MHz using HSE+PLL

- Flash read/write/erase operations

- UART communication at 115200 baud

- GPIO control for LED and USART1

- CRC-32 verification of application firmware

- Watchdog timer with 1s timeout

- Minimal LED blink application

- Safety mechanisms (no dynamic memory, MC/DC coverage)

I.3 Document Information

Project:           I-SCEET

Institution:       Personal project

Standard:          RTCA DO-178C §5 / SRS

I.4 Document Organization

This SRD contains five sections:

1. Introduction (purpose, scope, organization)

2. Referenced Documents (PDS, ARCH, HW, standards)

3. System Architecture (static/dynamic views)

4. High Level Requirements (HLRs)

5. PDS Coverage Summary (traceability)

I.5 DAL and Certification Context

The SRD is generated under DO-178C DAL A assumptions for safety-critical embedded systems. It establishes the baseline for M2 SDDD generation and aligns with PSAC/SDP lifecycle planning. All HLRs must be traceable to PDS items and verifiable through test/analysis per SRS.

I.6 Input Package Summary

| Input | Provided | Version/Notes |

|-------|----------|---------------|

| PDS   | Yes      | v3.1 (20 PDS items) |

| ARCH  | Yes      | v2.0 (static/dynamic architecture) |

| HW    | Yes      | STM32F303VCT6 datasheet + RM0316 |

| SRS   | Yes      | v2.0 (HLR/LLR standards) |

| PSAC  | Yes      | v4.0 (certification planning) |

| SDP   | Yes      | v4.0 (development process) |

SECTION II — REFERENCED DOCUMENTS

| Doc ID | Title | Version | Source | Applicability |
| --- | --- | --- | --- | --- |
| PDS | Product Design Specification | v3.1 | I-SCEET | PDS requirements |
| ARCH | System Architecture | v2.0 | I-SCEET | Architecture context |
| HW | STM32F303VCT6 Hardware | - | ST | Hardware constraints |
| SRS | Software Requirements Standards | v2.0 | I-SCEET | HLR/LLR format |
| PSAC | Plan for Software Aspects of Certification | v4.0 | I-SCEET | Certification planning |
| SDP | Software Development Plan | v4.0 | I-SCEET | Development process |
| DO-178C | DO-178C | ED-12C | RTCA | Certification standard |
| DO-330 | DO-330 | ED-330 | RTCA | Tool qualification |
| RM0316 | STM32F303 Reference Manual | Rev 10 | ST | Peripheral registers |

SECTION III — SYSTEM ARCHITECTURE

III.1 Static Architecture

| Block ID | Block Name | Responsibility | Inputs | Outputs | Related PDS IDs |
| --- | --- | --- | --- | --- | --- |
| B1 | Bootloader | Hardware init, CRC verification | Reset | UART banner, jump to app | PDS-BOOT-001, PDS-BOOT-002 |
| B2 | Clock Driver | Configure 72 MHz clock | HSE, PLL | Clock signals | PDS-CLK-001 |
| B3 | Flash Driver | Read/write/erase flash | Flash region | Flash status | PDS-FLASH-001-003 |
| B4 | UART Driver | Serial communication | UART data | UART output | PDS-UART-001-002 |
| B5 | GPIO Driver | LED and USART1 pins | GPIO config | LED state | PDS-GPIO-001 |
| B6 | CRC Module | Compute CRC-32 | Flash data | CRC result | PDS-CRC-001 |
| B7 | Watchdog | System reset on timeout | Timer | Reset signal | PDS-WDG-001-002 |
| B8 | SysTick | 1ms timebase | Timer | Tick count | PDS-TICK-001 |
| B9 | Application | LED blink | SysTick | UART message | PDS-APP-001-002 |
| Module | Purpose | Subsystem Code | Related Peripherals | PDS IDs |  |
| boot_manager | Bootloader coordination | BOOT | RCC, USART1, Flash | PDS-BOOT-001-003 |  |
| clk_driver | Clock configuration | CLK | RCC | PDS-CLK-001 |  |
| flash_driver | Flash operations | FLASH | Flash | PDS-FLASH-001-003 |  |
| uart_driver | UART communication | UART | USART1 | PDS-UART-001-002 |  |
| gpio_driver | GPIO control | GPIO | GPIOA | PDS-GPIO-001 |  |
| crc_module | CRC-32 computation | CRC | CRC | PDS-CRC-001 |  |
| wdg_driver | Watchdog management | WDG | IWDG | PDS-WDG-001-002 |  |
| systick_driver | Timebase | TICK | SysTick | PDS-TICK-001 |  |
| app_main | LED blink application | APP | GPIO, UART | PDS-APP-001-002 |  |
| Interface ID | Source | Destination | Data/Signal | Direction | Constraints |
| UART_TX | UART Driver | USART1 | 115200 8N1 | Output | 256-byte buffer |
| UART_RX | USART1 | UART Driver | Binary packets | Input | 1KB buffer |
| CRC_RESULT | CRC Module | Bootloader | 32-bit value | Output | Must match stored value |
| WDG_REFRESH | Watchdog | IWDG | Refresh signal | Output | Every 500ms |
| Region | Start Address | End Address | Size | Content | Protection |
| Vector Table | 0x08000000 | 0x080001FF | 512 B | IRQ vectors | Read-only |
| Bootloader Code | 0x08000200 | 0x08007FF7 | 31 KB | Bootloader | Read/execute |
| Firmware Metadata | 0x08007FF8 | 0x08007FFF | 8 B | App size/CRC | Read-only |
| Application Code | 0x08008000 | 0x0803FFFF | 224 KB | App code | Read/execute |
| Bootloader Stack | 0x20000000 | 0x20001FFF | 8 KB | Stack | Read/write |
| Driver Buffers | 0x20002000 | 0x20003FFF | 8 KB | UART buffers | Read/write |

III.1 Static Architecture

III.1.1 System Blocks / Modules

III.1.2 Software Functional Modules

III.1.3 External / Internal Interfaces

III.1.4 Memory Map

III.2 Dynamic Architecture

| State ID | State Name | Entry Condition | Exit Condition | Allowed Transitions |  |
| --- | --- | --- | --- | --- | --- |
| S1 | INIT_CLK | Power-on reset | Clock configured | S2 |  |
| S2 | INIT_PERIPH | Clock configured | Peripherals initialized | S3 |  |
| S3 | INIT_WDG | Peripherals initialized | Watchdog started | S4 |  |
| S4 | VERIFY_CRC | Watchdog started | CRC verified | S5 |  |
| S5 | JUMP_APP | CRC verified | Jump to application | - |  |
| S6 | ERROR_STATE | CRC failed | Watchdog reset | S1 |  |
| Flow ID | Source | Destination | Data/Control Item | Timing/Constraint | PDS IDs |
| F1 | Clock Driver | RCC | 72 MHz configuration | <10ms | PDS-CLK-001 |
| F2 | UART Driver | USART1 | 115200 baud setup | On init | PDS-UART-001 |
| F3 | Flash Driver | CRC Module | Application data | Before CRC | PDS-FLASH-003 |
| F4 | CRC Module | Bootloader | CRC result | <100ms | PDS-CRC-001 |
| F5 | Bootloader | UART Driver | Diagnostic message | <500ms | PDS-BOOT-002 |

Architecture

III.2.1 Boot Sequence

1. Power-on reset → RESET state

2. Initialize clock (72 MHz) → S1 INIT_CLK

3. Initialize SysTick, GPIO, UART, Flash, CRC → S2 INIT_PERIPH

4. Start watchdog → S3 INIT_WDG

5. Verify application CRC → S4 VERIFY_CRC

6. Jump to application or enter error state → S5 JUMP_APP/S6 ERROR_STATE

III.2.2 System State Machine

III.2.3 Data and Control Flows

III.2.4 ISR Handling Strategy

- USART1_IRQn (37): Priority 1, handles RX/TX interrupts for UART communication

- SysTick_IRQn (15): Priority 2, generates 1ms timebase

- IWDG_IRQn (not used): Watchdog is independent, no interrupt

- Shared data (UART buffers) protected by disabling interrupts during access

IV — HIGH LEVEL REQUIREMENTS

IV.1 — BOOT Requirements

IV.1.1 - Initialize Hardware in Order

<Activation Period: Once at boot>

IV.1.1.1 Input Data

None

IV.1.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| uart_banner | char[32] | [0;255] | "I-SCEET BOOTLOADER v3.1" |

IV.1.1.3 Local Data

None

IV.1.1.4 Requirement

HLR-BOOT-001

[COV.PDS-BOOT-001]

Initialize Hardware in Order

The software shall initialize hardware in the order RCC → SysTick → GPIO → UART → Flash → CRC → WDG and send a UART startup banner within 100 ms after UART initialization.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-BOOT-001 requires deterministic initialization sequence to ensure peripheral dependencies are met. UART banner confirms successful init.

[END_HLR-BOOT-001]

IV.1.2 - Verify Application CRC Before Execution

<Activation Period: Once at boot>

IV.1.2.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| app_crc | uint32_t | [0;0xFFFFFFFF] | CRC-32 of application code |

IV.1.2.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| diagnostic | char[16] | [0;255] | "CRC OK" or "CRC FAIL" |

IV.1.2.3 Local Data

None

IV.1.2.4 Requirement

HLR-BOOT-002

[COV.PDS-BOOT-002]

Verify Application CRC Before Execution

The software shall verify application CRC-32 before execution; it shall jump to 0x08008000 only if CRC verification succeeds; otherwise it shall enter a safe error state and send a UART diagnostic. Full init + verify + jump shall complete within 500 ms after power-on reset.

Upwards Traceability	Direct

Category	Safety

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	true

Rationale	PDS-BOOT-002 ensures firmware integrity before execution. Error state prevents corrupted code execution.

[END_HLR-BOOT-002]

IV.1.3 - Support Firmware Update via UART

<Activation Period: On event>

IV.1.3.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| rx_binary | uint8_t[224*1024] | [0;255] | Application binary |

IV.1.3.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| write_status | boolean_t | [0;1] | Flash write success flag |

IV.1.3.3 Local Data

None

IV.1.3.4 Requirement

HLR-BOOT-003

[COV.PDS-BOOT-003]

Support Firmware Update via UART

The software shall support firmware update through UART by receiving a new binary and writing it into the application flash region.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-BOOT-003 enables over-the-air updates. Flash write must be atomic and verified.

[END_HLR-BOOT-003]

IV.1.4 - Handle UART Transmission Timeout

<Activation Period: On event>

IV.1.4.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| start_time | uint32_t | [0;0xFFFFFFFF] | Transmission start tick |

IV.1.4.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| timeout_flag | boolean_t | [0;1] | Transmission timeout |

IV.1.4.3 Local Data

None

IV.1.4.4 Requirement

HLR-BOOT-004-D

[COV.[DERIVED]]

Handle UART Transmission Timeout

The software shall detect UART transmission timeout if 256-byte transmission exceeds 30 ms and abort the operation.

Upwards Traceability	Derived

Category	Performance

Context	Degraded

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	Derived from hardware constraints and PDS-UART-001 timing requirements.

[END_HLR-BOOT-004-D]

IV.2 — CLK Requirements

IV.2.1 - Configure 72 MHz System Clock

<Activation Period: Once at boot>

IV.2.1.1 Input Data

None

IV.2.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| clock_status | boolean_t | [0;1] | Clock config success flag |

IV.2.1.3 Local Data

None

IV.2.1.4 Requirement

HLR-CLK-001

[COV.PDS-CLK-001]

Configure 72 MHz System Clock

The software shall configure 72 MHz system clock using HSE 8 MHz + PLL x9, with AHB /1, APB1 /2 maximum 36 MHz, APB2 /1, within 10 ms of invocation.

Upwards Traceability	Direct

Category	Performance

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-CLK-001 requires stable high-speed clock for peripheral operation. APB1 must not exceed 36 MHz.

[END_HLR-CLK-001]

IV.3 — FLASH Requirements

IV.3.1 - Erase Flash Page

<Activation Period: On event>

IV.3.1.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| page_addr | uint32_t | [0x08008000;0x0803FFFF] | Flash page address |

IV.3.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| erase_status | boolean_t | [0;1] | Page erase success flag |

IV.3.1.3 Local Data

None

IV.3.1.4 Requirement

HLR-FLASH-001

[COV.PDS-FLASH-001]

Erase Flash Page

The software shall erase any page in the application region 0x08008000–0x0803FFFF within 50 ms per page.

Upwards Traceability	Direct

Category	Performance

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-FLASH-001 ensures flash is ready for new firmware. Page erase must be atomic.

[END_HLR-FLASH-001]

IV.3.2 - Write Half-Word to Flash

<Activation Period: On event>

IV.3.2.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| write_addr | uint32_t | [0x08008000;0x0803FFFF] | Target address |
| write_data | uint16_t | [0;0xFFFF] | Data to write |

IV.3.2.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| verify_result | boolean_t | [0;1] | Readback matches write |

IV.3.2.3 Local Data

None

IV.3.2.4 Requirement

HLR-FLASH-002

[COV.PDS-FLASH-002]

Write Half-Word to Flash

The software shall write half-words, 16-bit, to any address in the application region and verify each write by readback comparison.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-FLASH-002 ensures data integrity during firmware update. Write must be verified.

[END_HLR-FLASH-002]

IV.4 — UART Requirements

IV.4.1 - Transmit Null-Terminated Strings

<Activation Period: Continuous>

IV.4.1.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| tx_buffer | uint8_t[256] | [0;255] | String to transmit |

IV.4.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| tx_complete | boolean_t | [0;1] | Transmission success |

IV.4.1.3 Local Data

None

IV.4.1.4 Requirement

HLR-UART-001

[COV.PDS-UART-001]

Transmit Null-Terminated Strings

The software shall configure USART1 at 115200 baud 8N1 and transmit null-terminated strings up to 256 characters; 256-byte transmission shall complete within 30 ms.

Upwards Traceability	Direct

Category	Performance

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-UART-001 enables diagnostics and firmware update progress reporting.

[END_HLR-UART-001]

IV.5 — GPIO Requirements

IV.5.1 - Configure GPIO Pins

<Activation Period: Once at boot>

IV.5.1.1 Input Data

None

IV.5.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| gpio_config | boolean_t | [0;1] | Pin config success |

IV.5.1.3 Local Data

None

IV.5.1.4 Requirement

HLR-GPIO-001

[COV.PDS-GPIO-001]

Configure GPIO Pins

The software shall configure PA5 as push-pull 2 MHz output for LED set/reset/toggle operations, PA9 as USART1 TX alternate-function output, and PA10 as USART1 RX alternate-function input.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-GPIO-001 defines LED control and USART1 pin usage.

[END_HLR-GPIO-001]

IV.6 — CRC Requirements

IV.6.1 - Compute CRC-32 for Application Code

<Activation Period: Once at boot>

IV.6.1.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| start_addr | uint32_t | [0x08008000;0x0803FFFF] | Start address |
| length | uint32_t | [0;0x3FFFF] | Bytes to compute |

IV.6.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| crc_result | uint32_t | [0;0xFFFFFFFF] | Computed CRC |

IV.6.1.3 Local Data

None

IV.6.1.4 Requirement

HLR-CRC-001

[COV.PDS-CRC-001]

Compute CRC-32 for Application Code

The software shall compute CRC-32 using polynomial 0x04C11DB7 over a memory range using the hardware CRC unit, reset before each computation; it shall complete 224 KB in less than 100 ms, with expected value stored at 0x08007FFC.

Upwards Traceability	Direct

Category	Safety

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	true

Rationale	PDS-CRC-001 ensures application code integrity before execution.

[END_HLR-CRC-001]

IV.7 — TICK Requirements

IV.7.1 - Generate 1ms Timebase

<Activation Period: Continuous>

IV.7.1.1 Input Data

None

IV.7.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| tick_count | uint32_t | [0;0xFFFFFFFF] | Current tick value |

IV.7.1.3 Local Data

None

IV.7.1.4 Requirement

HLR-TICK-001

[COV.PDS-TICK-001]

Generate 1ms Timebase

The software shall generate a 1 ms interrupt at 72 MHz and provide HAL_GetTick()/HAL_Delay()-equivalent services; tick counter shall not overflow within 49 days of continuous operation.

Upwards Traceability	Direct

Category	Performance

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-TICK-001 provides timing reference for watchdog refresh and application.

[END_HLR-TICK-001]

IV.8 — WDG Requirements

IV.8.1 - Configure IWDG with 1s Timeout

<Activation Period: Once at boot>

IV.8.1.1 Input Data

None

IV.8.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| wdg_status | boolean_t | [0;1] | Watchdog config success |

IV.8.1.3 Local Data

None

IV.8.1.4 Requirement

HLR-WDG-001

[COV.PDS-WDG-001]

Configure IWDG with 1s Timeout

The software shall configure IWDG with 1 s timeout from LSI during bootloader initialization, provide refresh service, refresh at least every 500 ms, and never stop IWDG once started.

Upwards Traceability	Direct

Category	Safety

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	true

Rationale	PDS-WDG-001 ensures system reset on software hang.

[END_HLR-WDG-001]

IV.8.2 - Reset on Watchdog Timeout

<Activation Period: On event>

IV.8.2.1 Input Data

None

IV.8.2.2 Output Data

None

IV.8.2.3 Local Data

None

IV.8.2.4 Requirement

HLR-WDG-002

[COV.PDS-WDG-002]

Reset on Watchdog Timeout

If IWDG expires due to system hang, the MCU shall perform automatic system reset.

Upwards Traceability	Direct

Category	Safety

Context	Failure

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	true

Rationale	PDS-WDG-002 ensures recovery from unresponsive states.

[END_HLR-WDG-002]

IV.9 — APP Requirements

IV.9.1 - Toggle LED at 1Hz

<Activation Period: Continuous>

IV.9.1.1 Input Data

None

IV.9.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| led_state | boolean_t | [0;1] | LED ON/OFF status |

IV.9.1.3 Local Data

None

IV.9.1.4 Requirement

HLR-APP-001

[COV.PDS-APP-001]

Toggle LED at 1Hz

The software shall toggle LED on PA5 at 1 Hz, 500 ms ON/OFF, using SysTick and shall run indefinitely under normal conditions as visual validation that M1–M6 produced working code.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-APP-001 validates end-to-end toolchain output.

[END_HLR-APP-001]

IV.9.2 - Send Startup Message and Refresh Watchdog

<Activation Period: Once at boot>

IV.9.2.1 Input Data

None

IV.9.2.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| uart_message | char[18] | [0;255] | "I-SCEET APP RUNNING" |

IV.9.2.3 Local Data

None

IV.9.2.4 Requirement

HLR-APP-002

[COV.PDS-APP-002]

Send Startup Message and Refresh Watchdog

The software shall send 'I-SCEET APP RUNNING' once at startup through UART and refresh IWDG every 500 ms.

Upwards Traceability	Direct

Category	Functional

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-APP-002 confirms application execution and prevents watchdog reset.

[END_HLR-APP-002]

IV.10 — SYS Requirements

IV.10.1 - Prevent Execution of Corrupted Firmware

<Activation Period: Once at boot>

IV.10.1.1 Input Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| crc_result | uint32_t | [0;0xFFFFFFFF] | Computed CRC |

IV.10.1.2 Output Data

| Data Name | Type | Range | Description |
| --- | --- | --- | --- |
| jump_allowed | boolean_t | [0;1] | Jump to app permitted |

IV.10.1.3 Local Data

None

IV.10.1.4 Requirement

HLR-SYS-001

[COV.PDS-SYS-001]

Prevent Execution of Corrupted Firmware

The software shall detect application firmware corruption before execution using CRC-32 and shall never execute corrupted or unverified application code.

Upwards Traceability	Direct

Category	Safety

Context	Nominal

Verification Method	Test

Security Assurance Level	N/A

Safety Effect	true

Rationale	PDS-SYS-001 ensures safety by preventing execution of invalid code.

[END_HLR-SYS-001]

IV.10.2 - No Dynamic Memory Allocation

<Activation Period: Continuous>

IV.10.2.1 Input Data

None

IV.10.2.2 Output Data

None

IV.10.2.3 Local Data

None

IV.10.2.4 Requirement

HLR-SYS-002

[COV.PDS-SYS-002]

No Dynamic Memory Allocation

The software shall not use dynamic memory allocation or recursion in bootloader or drivers and shall handle all peripheral initialization failures gracefully with no undefined behavior.

Upwards Traceability	Direct

Category	Resource

Context	Nominal

Verification Method	Static Analysis

Security Assurance Level	N/A

Safety Effect	false

Rationale	PDS-SYS-002 ensures stack safety and deterministic behavior.

[END_HLR-SYS-002]

IV.SUMMARY — PDS Coverage

Total HLRs generated: 17

Standard HLRs: 16

Derived HLRs (-D): 1