SOFTWARE DESIGN STANDARDS

SDS-001

Intelligent Safety-Critical Engineering Environment Toolchain

Rules for Software Architecture and Design — Governs M2

| Field | Value |
| --- | --- |
| Document ID | SDS-001 |
| Version | 1.0 |
| Status | DRAFT |
| Date | 2026-04-19 |
| Author | Hatem ADOUANI |
| Project | I-SCEET |
| Institution | Capgemini Engineering / ESPRIT |
| Supervisor | [Supervisor Name] |
| Governs | M2 (Software Architecture + LLR generation) → SDDD-001 |
| Standard | RTCA DO-178C §6 / EUROCAE ED-12C |

# 1. Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-04-19 | Hatem ADOUANI | Initial release — defines software architecture and design rules for M2 |

# 2. Introduction

## 2.1 Purpose

This Software Design Standards document (SDS-001) defines the rules, constraints, and quality criteria that the software architecture and design produced by M2 must satisfy. It governs the structure of SDDD-001, covering both the Software Architecture section (SDDD-001 §1) and the Low Level Requirements section (SDDD-001 §2).

These rules shall be injected as constraints into the M2 AI module prompt and shall be applied by the independent reviewer during verification of SDDD-001.

## 2.2 Scope

This document covers: software architecture rules, module decomposition criteria, interface specification standards, data flow and control flow constraints, complexity limits, and coupling/cohesion rules. It applies to all architecture and design artifacts in SDDD-001 regardless of the target project.

## 2.3 Referenced Documents

- RTCA DO-178C §6 — Software Design Process

- PSAC-TOOLCHAIN-001 — Plan for Software Aspects of Certification

- SDP-TOOLCHAIN-001 — Software Development Plan

- SVP-TOOLCHAIN-001 — Software Verification Plan

- SRS-001 — Software Requirements Standards (governs LLR writing rules)

- SCS-001 — Software Code Standards (governs M3 output)

# 3. SDDD-001 Document Structure

## 3.1 Mandatory Sections

SDDD-001 generated by M2 shall contain exactly the following two sections:

| Section | Title | Content | Generated by |
| --- | --- | --- | --- |
| §1 | Software Architecture | Module decomposition, interfaces, data flows, control flows, memory map, scheduler design | M2 (AI-generated from SRD-001 HLRs) |
| §2 | Low Level Requirements | Complete set of LLR-[SUBSYSTEM]-[NNN] blocks per SRS-001 rules | M2 (AI-generated from SRD-001 HLRs) |

## 3.2 Section 1 — Software Architecture Mandatory Content

The Software Architecture section of SDDD-001 shall contain all of the following sub-sections:

| Sub-section | Content Required |
| --- | --- |
| 1.1 — Architecture Overview | High-level description of the software structure; number of modules; execution model (bare-metal, RTOS, superloop) |
| 1.2 — Module Decomposition | List of all software modules with name, purpose, and corresponding HLR(s) they implement |
| 1.3 — Interface Definitions | All module-to-module interfaces: function signatures, shared data structures, message formats |
| 1.4 — Data Flow Diagram | Description of how data moves between modules: sources, transformations, destinations |
| 1.5 — Control Flow Description | Execution sequence, interrupt priorities, task scheduling, initialization order |
| 1.6 — Memory Map | RAM and Flash memory allocation per module: start address, size, access rights |
| 1.7 — Error Handling Strategy | How errors propagate between modules; error codes definition; safe state definition |
| 1.8 — Hardware Abstraction Layer | HAL module list; mapping of HAL functions to hardware registers and peripherals |

# 4. Module Decomposition Rules

## 4.1 Module Definition

A software module is the primary unit of decomposition in the I-SCEET architecture. Each module shall:

- Implement one or more related LLRs from a single subsystem (BOOT, FLASH, UART, CLK, GPIO, CRC, SYS)

- Correspond to exactly one source file pair: [module_name].c and [module_name].h

- Have a single, clearly stated purpose expressible in one sentence

- Be independently compilable without modifying other modules

## 4.2 Module Naming Convention

| Level | Convention | Example |
| --- | --- | --- |
| Module name | lowercase_snake_case | flash_driver, uart_driver, boot_manager |
| Source file | [module_name].c | flash_driver.c |
| Header file | [module_name].h | flash_driver.h |
| Init function | [module_name]_init() | flash_driver_init() |
| De-init function | [module_name]_deinit() | uart_driver_deinit() |
| Process function | [module_name]_process() | boot_manager_process() |
| Get function | [module_name]_get_[attribute]() | crc_module_get_result() |
| Set function | [module_name]_set_[attribute]() | uart_driver_set_baudrate() |

## 4.3 Module Decomposition Criteria

M2 shall decompose the software into modules following these criteria:

| Criterion | Rule | Rationale |
| --- | --- | --- |
| Single Responsibility | Each module implements requirements from exactly one subsystem code (BOOT, FLASH, UART, etc.) | Minimizes coupling; simplifies traceability to LLRs |
| High Cohesion | All functions in a module serve the same hardware peripheral or functional domain | Easier to test, review, and maintain |
| Low Coupling | Modules communicate only through defined interfaces in their .h headers — no direct access to another module's internal variables | Prevents hidden dependencies; enables independent testing |
| Complete Coverage | Every HLR in SRD-001 shall be implemented by at least one module | Ensures no HLR is left unimplemented before coding starts |
| Testability | Each module shall be independently testable with a module-level test harness (for M4 LLT generation) | Enables structural coverage measurement per DO-178C §9 |

## 4.4 Module Size Limits

| Metric | Limit | Rationale |
| --- | --- | --- |
| Maximum functions per module | ≤ 20 functions | Keeps module scope manageable and reviewable |
| Maximum source file size | ≤ 500 lines | Limits review effort; enforces single responsibility |
| Maximum header file size | ≤ 100 lines | Interface should be minimal and clean |
| Maximum module dependencies | ≤ 5 other modules | Prevents dependency explosion; limits coupling |
| Minimum functions per module | ≥ 2 functions | Ensures init + at least one functional operation |

# 5. Interface Design Rules

## 5.1 Public Interface Rules

The public interface of each module (exposed in its .h header file) shall follow these rules:

- All public functions shall be declared in the module header file — no undeclared public functions

- All public data structures shall be defined in the header file — no implicit type sharing

- No global variables shall be declared as public — use getter/setter functions instead

- All function parameters shall be explicitly typed — no void* without justification

- All functions returning error status shall use the project error code type (see §5.3)

## 5.2 Function Interface Standard

Every public function defined in SDDD-001 and generated by M3 shall follow this interface pattern:

[ReturnType] [module_name]_[action]([ParamType] [param_name], ...);

Complete example for the STM32F303 bootloader:

/* flash_driver.h */

ISCEET_Status_t flash_driver_init(void);

ISCEET_Status_t flash_driver_erase_page(uint32_t page_address);

ISCEET_Status_t flash_driver_write(uint32_t address, const uint8_t* data, uint32_t length);

ISCEET_Status_t flash_driver_read(uint32_t address, uint8_t* buffer, uint32_t length);

ISCEET_Status_t flash_driver_verify(uint32_t address, const uint8_t* expected, uint32_t length);

## 5.3 Error Code Standard

All modules shall use the following project-wide error code type defined in a shared header (isceet_types.h):

typedef enum {

ISCEET_OK           = 0x00,   /* Operation successful */

ISCEET_ERROR        = 0x01,   /* Generic error */

ISCEET_TIMEOUT      = 0x02,   /* Operation timed out */

ISCEET_INVALID_PARAM= 0x03,   /* Invalid parameter passed */

ISCEET_HW_FAULT     = 0x04,   /* Hardware peripheral failure */

ISCEET_CRC_FAIL     = 0x05,   /* CRC verification failed */

ISCEET_FLASH_ERROR  = 0x06,   /* Flash operation failed */

ISCEET_BUSY         = 0x07,   /* Resource currently busy */

} ISCEET_Status_t;

## 5.4 Inter-Module Communication Rules

- Modules shall communicate only through function calls defined in public headers

- Shared data between modules shall use dedicated data structures — no raw global variables

- Callback functions shall be registered through explicit registration functions — no function pointer globals

- Interrupt Service Routines (ISRs) shall only set flags or write to ring buffers — no complex logic in ISRs

# 6. Complexity Limits

## 6.1 Cyclomatic Complexity

Cyclomatic complexity (CC) measures the number of linearly independent paths through a function. High CC increases MC/DC coverage effort and review burden. The following limits apply:

| Limit | Value | Action if Exceeded |
| --- | --- | --- |
| Maximum CC per function (normal) | ≤ 10 | Refactor into smaller functions before M3 generates code |
| Maximum CC per function (safety-critical ISR) | ≤ 4 | ISRs must be minimal — raise PR if exceeded |
| Maximum CC per module (sum) | ≤ 50 | Module must be split into two or more modules |
| Target CC per function (ideal) | ≤ 5 | Preferred for DAL A — minimizes MC/DC test effort |

## 6.2 Nesting Depth Limits

| Context | Maximum Nesting Depth | Rationale |
| --- | --- | --- |
| General functions | ≤ 4 levels | Deeper nesting creates complex MC/DC conditions |
| ISR functions | ≤ 2 levels | ISRs must execute fast with minimal branching |
| Switch statements | ≤ 3 cases of nesting inside each case | Prevents unmaintainable state machines |

## 6.3 Function Length Limits

| Metric | Limit | Exception |
| --- | --- | --- |
| Maximum lines per function (code only) | ≤ 60 lines | State machines may go to 100 lines with justification |
| Maximum parameters per function | ≤ 6 params | Use a struct if more than 6 parameters are needed |
| Maximum return points per function | ≤ 2 returns | Prefer single exit; early return for error only |
| Maximum local variables per function | ≤ 10 vars | More suggests function should be split |

## 6.4 Architecture-Level Complexity

| Metric | Limit |
| --- | --- |
| Maximum number of software modules | ≤ 20 modules for the STM32F303 test project |
| Maximum call depth (call stack levels) | ≤ 8 levels — must be analyzable statically |
| Maximum number of interrupts | ≤ 8 active interrupt sources |
| Maximum interrupt nesting allowed | 0 — no nested interrupts for DAL A |
| Maximum shared resources between modules | ≤ 3 shared resources (protected by flags or mutexes) |

# 7. Data Design Rules

## 7.1 Data Type Rules

- All data types shall use fixed-width types from stdint.h: uint8_t, uint16_t, uint32_t, int8_t, int16_t, int32_t

- 'int', 'long', 'short' without width qualifier are FORBIDDEN — size is platform-dependent

- 'float' and 'double' are FORBIDDEN unless explicitly justified and reviewed — use fixed-point arithmetic instead

- Boolean values shall use the project-defined ISCEET_Bool_t type or stdint uint8_t with defined TRUE/FALSE macros

- Pointer arithmetic is FORBIDDEN except in HAL and memory driver modules with explicit justification

## 7.2 Global Data Rules

- Global variables are FORBIDDEN in all modules except the following justified cases:

- → ISR flag variables (volatile uint8_t) — one per interrupt source maximum

- → Module state variables (static, not extern) — not visible outside the module

- All module-internal persistent state shall use static local variables — never extern globals

- No dynamic memory allocation (malloc, calloc, free) is permitted anywhere in the design

## 7.3 Memory Map Rules

The software architecture in SDDD-001 §1.6 shall define a memory map for the STM32F303 with the following structure:

| Region | Address Range | Size | Content |
| --- | --- | --- | --- |
| Vector Table | 0x08000000 — 0x080001FF | 512 B | ARM Cortex-M4 interrupt vector table |
| Bootloader Code | 0x08000200 — 0x08003FFF | ~15 KB | M3-generated bootloader source compiled |
| Firmware Metadata | 0x08003FFC — 0x08003FFF | 4 B | CRC-32 value of application firmware |
| Application Code | 0x08004000 — 0x0807FFFF | ~240 KB | Target application firmware image |
| RAM — Stack | 0x20000000 — 0x20001FFF | 8 KB | Bootloader call stack — grows downward |
| RAM — Data/BSS | 0x20002000 — 0x20003FFF | 8 KB | Module static variables and buffers |

# 8. Control Flow and Execution Model Rules

## 8.1 Execution Model

The STM32F303 bootloader shall use a bare-metal superloop execution model. The architecture generated by M2 shall comply with the following execution model:

/* Mandatory boot sequence — M2 must generate LLRs for each step */

1. Reset handler → copy .data, zero .bss, call SystemInit()

2. CLK module    → configure HSE + PLL to 72MHz

3. GPIO module   → configure required pins

4. UART module   → initialize at 115200 baud for debug/update

5. FLASH module  → initialize flash controller

6. CRC module    → verify firmware CRC-32

7. BOOT module   → if CRC OK → jump to application

→ if CRC FAIL → enter update/error mode

## 8.2 Interrupt Architecture Rules

- ISR functions shall be defined only in the module responsible for the corresponding peripheral

- ISRs shall execute in less than 10µs — this shall be documented as a design constraint in SDDD-001

- ISRs shall only set a volatile flag or write to a ring buffer — no function calls from ISR except HAL primitives

- All ISR priority levels shall be defined in SDDD-001 §1.5 with justification

- No two ISRs shall share the same priority unless explicitly justified

## 8.3 Initialization Order

The initialization order of all modules shall be explicitly defined in SDDD-001 §1.5 and shall:

- Be consistent with hardware dependency order (clock before peripherals, peripherals before drivers)

- Assign a priority number to each module init function: lower number = earlier initialization

- Guarantee that no module's init function calls another module's non-init function

# 9. Architecture and Design Review Checklist

The independent reviewer shall use this checklist to verify SDDD-001 §1 (Architecture) before M3 is triggered:

## 9.1 Architecture Checklist

| # | Check Item | Pass / Fail |
| --- | --- | --- |
| A01 | SDDD-001 §1 contains all 8 mandatory sub-sections (§1.1 to §1.8) |  |
| A02 | Every HLR from SRD-001 is implemented by at least one identified module |  |
| A03 | All modules follow the naming convention: lowercase_snake_case |  |
| A04 | No module exceeds size limits (≤20 functions, ≤500 lines source) |  |
| A05 | Module dependencies are documented and no circular dependency exists |  |
| A06 | All public interfaces are defined in header files — no undeclared public functions |  |
| A07 | ISCEET_Status_t is used as return type for all functions that can fail |  |
| A08 | No global variables declared except justified ISR flags and static module state |  |
| A09 | No dynamic memory allocation (malloc/free) used anywhere |  |
| A10 | Memory map in §1.6 covers all memory regions with correct addresses for STM32F303 |  |
| A11 | ISR priority table in §1.5 is complete and no two ISRs share the same priority |  |
| A12 | Initialization order in §1.5 is consistent with hardware dependencies |  |
| A13 | Error handling strategy in §1.7 covers all failure modes identified in SRD-001 |  |
| A14 | No module has more than 5 dependencies on other modules |  |
| A15 | Architecture is consistent with System Architecture provided in SRD-001 §1 |  |

## 9.2 Design (LLR) Checklist

In addition to the SRS-001 LLR checklist, the following design-specific items shall be verified:

| # | Check Item | Pass / Fail |
| --- | --- | --- |
| D01 | Every LLR references a function that exists in the module decomposition in §1.2 |  |
| D02 | No LLR function exceeds cyclomatic complexity limit (≤10) |  |
| D03 | No LLR function exceeds nesting depth limit (≤4 levels) |  |
| D04 | No LLR function exceeds line count limit (≤60 lines) |  |
| D05 | All data types in LLRs use fixed-width types (uint8_t, uint16_t, uint32_t...) |  |
| D06 | No LLR uses float or double without explicit justification |  |
| D07 | All timing values in LLRs are in ms or µs with target hardware reference |  |
| D08 | All memory addresses in LLRs are in hexadecimal format |  |
| D09 | All register names in LLRs match the STM32F303 reference manual exactly |  |
| D10 | Error handling is specified for every LLR function that can fail |  |

# 10. Examples — STM32F303 Bootloader

## 10.1 Correct Module Decomposition

For the STM32F303 bootloader, M2 shall generate the following module decomposition in SDDD-001 §1.2:

| Module Name | Source File | Purpose | Implements HLRs |
| --- | --- | --- | --- |
| boot_manager | boot_manager.c/.h | Main bootloader sequencing: init, verify, jump | HLR-BOOT-001, HLR-BOOT-002, HLR-BOOT-003 |
| clk_driver | clk_driver.c/.h | RCC configuration: HSE enable, PLL setup, AHB/APB prescalers | HLR-CLK-001 |
| flash_driver | flash_driver.c/.h | Internal flash: erase, write, read, verify operations | HLR-FLASH-001, HLR-FLASH-002 |
| uart_driver | uart_driver.c/.h | USART1 configuration, transmit/receive, interrupt handling | HLR-UART-001, HLR-UART-002 |
| gpio_driver | gpio_driver.c/.h | GPIO pin configuration, read, write for boot/status pins | HLR-GPIO-001 |
| crc_module | crc_module.c/.h | CRC-32 computation using STM32 hardware CRC unit | HLR-CRC-001 |
| sys_handler | sys_handler.c/.h | Watchdog init, error safe state, system reset management | HLR-SYS-001, HLR-SYS-002 |
| isceet_types | isceet_types.h | Shared type definitions: ISCEET_Status_t, ISCEET_Bool_t | N/A — shared types |

## 10.2 Correct LLR Example — Design Compliant

| Field | Value |
| --- | --- |
| ID | LLR-CLK-001 |
| Title | PLL Clock Configuration to 72MHz |
| Statement | The software shall configure the RCC peripheral to enable the HSE oscillator at 8MHz, configure the PLL with multiplier x9 (PLLMUL=0b0111), set PLLSRC to HSE, and select PLL as SYSCLK source, resulting in a system clock of 72MHz. |
| Rationale | HLR-CLK-001 requires 72MHz system clock; STM32F303 RM0316 §8.3 defines the PLL configuration registers. |
| Parent | HLR-CLK-001 |
| Priority | SHALL |
| Status | DRAFT |
| Verification | Test — measure SYSCLK via MCO pin output and oscilloscope, expected 72MHz ±1% |

## 10.3 Forbidden Design Patterns

| Pattern | Why Forbidden | Correct Alternative |
| --- | --- | --- |
| extern uint32_t global_flash_address; | Global variable — visible across modules, untraceable writes | Use flash_driver_get_address() getter function |
| void* data parameter | Type-unsafe — hides actual type from reviewer | Use explicit typed parameter: uint8_t* data |
| malloc(sizeof(FlashBuffer_t)) | Dynamic allocation — heap fragmentation, non-deterministic | Declare static FlashBuffer_t buf; in module scope |
| while(!(FLASH->SR & FLASH_SR_BSY)) {} | Infinite busy-wait — no timeout protection | Add timeout counter; return ISCEET_TIMEOUT if exceeded |
| Nested ISR calls | Non-deterministic timing; stack overflow risk | Set flag in ISR; handle in main loop |

# 11. Approvals

This document requires the following approvals before SOI #1 and before M2 is executed:

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Author | Hatem ADOUANI |  |  |
| Project Supervisor | [Supervisor Name] |  |  |
| Quality Assurance | [QA Name] |  |  |
| Certification Authority | [Authority Rep] |  |  |