PRODUCT DESIGN SPECIFICATION

PDS

STM32F303 Bootloader + Drivers + Minimal Application

I-SCEET Toolchain Validation Test Project

| Field | Value |
| --- | --- |
| Document ID | PDS-STM32F303-001 |
| Version | 1.0 |
| Status | DRAFT |
| Date | 2026-04-19 |
| Author | Hatem ADOUANI |
| Project | I-SCEET — STM32F303 Test Case |
| Institution | Capgemini Engineering / ESPRIT |
| Target HW | STM32F303VCT6 — ARM Cortex-M4 @ 72MHz, 256KB Flash, 40KB RAM |
| DAL Level | A — Catastrophic (DO-178C / DO-254) |
| Purpose | Primary input to M1 (HLR generation) — feeds I-SCEET toolchain |
| Standard | RTCA DO-178C / ARP4754A / ARP4761 |

# 1. Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-04-19 | Hatem ADOUANI | Initial release — full PDS for STM32F303 test project |

# 2. Introduction

## 2.1 Purpose

This Product Design Specification (PDS) defines the functional, performance, interface, and safety requirements for the STM32F303 embedded software system used as the validation test case for the I-SCEET toolchain. This document is the primary input to M1 (HLR Generator) and shall be provided alongside the Hardware Document and System Architecture before M1 is triggered.

## 2.2 System Description

The STM32F303 test project consists of three software layers:

- Bootloader — validates firmware integrity, initializes hardware, and jumps to the application

- Drivers — peripheral abstraction layer for UART, Flash, GPIO, CRC, SysTick, WDG, and RCC

- Minimal Application — a simple LED blink application that confirms successful boot and validates the full toolchain end-to-end

## 2.3 Toolchain Role

This PDS feeds the I-SCEET toolchain as follows:

| PDS Section | Fed to Module | Produces |
| --- | --- | --- |
| §4 — Bootloader requirements | M1 | HLR-BOOT-xxx in SRD-001 |
| §5 — Driver requirements | M1 | HLR-FLASH/UART/GPIO/CRC/CLK/SYS-xxx in SRD-001 |
| §6 — Application requirements | M1 | HLR-APP-xxx in SRD-001 |
| §7 — Safety requirements | M1 | HLR-SYS-xxx (safety) in SRD-001 |
| §8 — Performance requirements | M1 | Timing/resource HLRs in SRD-001 |

## 2.4 Referenced Documents

- STM32F303VCT6 Reference Manual — RM0316 (STMicroelectronics)

- STM32F303VCT6 Datasheet (STMicroelectronics)

- RTCA DO-178C — Software Considerations in Airborne Systems

- SAE ARP4754A — Guidelines for Development of Civil Aircraft and Systems

- SAE ARP4761 — Guidelines for Conducting Safety Assessments

- PSAC-TOOLCHAIN-001 — Plan for Software Aspects of Certification

# 3. Hardware Overview

## 3.1 Target Microcontroller

| Parameter | Value |
| --- | --- |
| MCU | STM32F303VCT6 |
| Core | ARM Cortex-M4F with FPU |
| Clock | Up to 72MHz (HSE 8MHz + PLL x9) |
| Flash | 256KB internal flash — 0x08000000 to 0x0803FFFF |
| RAM | 40KB SRAM — 0x20000000 to 0x20009FFF |
| Package | LQFP100 |
| Operating voltage | 2.0V to 3.6V |
| Operating temp | -40°C to +85°C (industrial grade) |

## 3.2 Peripheral Mapping

| Peripheral | Instance | Pins | Purpose in Project |
| --- | --- | --- | --- |
| UART | USART1 | PA9 (TX), PA10 (RX) | Debug output + firmware update interface |
| Flash | Internal | N/A | Firmware storage and bootloader execution |
| GPIO | GPIOA | PA5 (LED) | Status LED for application blink validation |
| CRC | CRC unit | N/A (hardware unit) | CRC-32 firmware integrity verification |
| SysTick | SysTick | N/A (core peripheral) | 1ms system timebase for all timing functions |
| WDG | IWDG | N/A (internal) | Independent watchdog — system safety guardian |
| RCC | RCC | OSC_IN/OSC_OUT (PD0/PD1) | Clock tree configuration — HSE + PLL |

## 3.3 Memory Map

| Region | Start Address | End Address | Size | Content |
| --- | --- | --- | --- | --- |
| Vector Table | 0x08000000 | 0x080001FF | 512 B | ARM Cortex-M4 interrupt vectors |
| Bootloader Code | 0x08000200 | 0x08007FFF | ~31 KB | Bootloader + all drivers compiled |
| Firmware Metadata | 0x08007FF8 | 0x08007FFF | 8 B | App size (4B) + CRC-32 (4B) |
| Application Code | 0x08008000 | 0x0803FFFF | ~224 KB | Minimal LED blink application |
| RAM — Stack | 0x20000000 | 0x20001FFF | 8 KB | Bootloader stack (grows down) |
| RAM — Heap/BSS | 0x20002000 | 0x20009FFF | 32 KB | Driver buffers + app variables |

# 4. Bootloader Requirements

## 4.1 Functional Requirements

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-BOOT-001 | The bootloader shall initialize all required hardware peripherals in the correct order: RCC → SysTick → GPIO → UART → Flash → CRC → WDG | SHALL | Test |
| PDS-BOOT-002 | The bootloader shall verify the application firmware CRC-32 before transferring execution to the application | SHALL | Test |
| PDS-BOOT-003 | The bootloader shall jump to the application start address (0x08008000) if and only if CRC-32 verification succeeds | SHALL | Test |
| PDS-BOOT-004 | The bootloader shall enter a safe error state and send a diagnostic message via UART if CRC-32 verification fails | SHALL | Test |
| PDS-BOOT-005 | The bootloader shall support firmware update via UART by receiving a new firmware binary and writing it to the application flash region | SHALL | Test |
| PDS-BOOT-006 | The bootloader shall complete the full boot sequence (init + CRC verify + jump) within 500ms of power-on reset | SHALL | Test |
| PDS-BOOT-007 | The bootloader shall send a startup banner message via UART within 100ms of UART initialization | SHOULD | Test |

## 4.2 Boot Sequence

The bootloader shall execute the following mandatory sequence on every power-on or reset event:

| Step | Action | Success Condition | Failure Action |
| --- | --- | --- | --- |
| 1 | RCC Init — configure HSE + PLL to 72MHz | SYSCLK = 72MHz confirmed | Halt — system cannot operate at wrong frequency |
| 2 | SysTick Init — configure 1ms timebase | SysTick interrupt fires at 1ms | Halt — all timing functions depend on SysTick |
| 3 | GPIO Init — configure PA5 as output, PA9/PA10 | Pins configured without fault | Continue with warning via UART |
| 4 | UART Init — configure USART1 at 115200 baud | UART ready to transmit | Continue without UART (silent boot) |
| 5 | Flash Init — unlock and configure flash | Flash controller ready | Halt — cannot verify or update firmware |
| 6 | CRC Init — reset hardware CRC unit | CRC unit ready | Halt — cannot verify firmware integrity |
| 7 | WDG Init — start IWDG with 1 second timeout | IWDG running | Halt — safety guardian must be active |
| 8 | CRC Verify — compute and compare firmware CRC | CRC matches stored value | Enter safe error state, send UART diagnostic |
| 9 | Jump to App — set MSP, jump to 0x08008004 | Application executes | N/A — point of no return |

# 5. Driver Requirements

## 5.1 RCC Driver (clk_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-CLK-001 | The RCC driver shall configure the system clock to 72MHz using HSE oscillator (8MHz) and PLL with multiplier x9 | SHALL | Test |
| PDS-CLK-002 | The RCC driver shall configure AHB prescaler to 1 (72MHz), APB1 prescaler to 2 (36MHz), APB2 prescaler to 1 (72MHz) | SHALL | Analysis |
| PDS-CLK-003 | The RCC driver shall enable clocks for all required peripherals: GPIOA, USART1, CRC, FLASH, IWDG | SHALL | Review |
| PDS-CLK-004 | The RCC driver shall complete clock configuration within 10ms of invocation | SHALL | Test |

## 5.2 Flash Driver (flash_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-FLASH-001 | The flash driver shall support page erase operations for any page in the application region (0x08008000 to 0x0803FFFF) | SHALL | Test |
| PDS-FLASH-002 | The flash driver shall support half-word (16-bit) write operations to any address in the application region | SHALL | Test |
| PDS-FLASH-003 | The flash driver shall support read operations from any address in the entire flash memory (0x08000000 to 0x0803FFFF) | SHALL | Test |
| PDS-FLASH-004 | The flash driver shall verify written data by reading back and comparing after every write operation | SHALL | Test |
| PDS-FLASH-005 | The flash driver shall complete a full page erase within 50ms | SHALL | Test |
| PDS-FLASH-006 | The flash driver shall return an error code if any flash operation fails or times out | SHALL | Test |

## 5.3 UART Driver (uart_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-UART-001 | The UART driver shall configure USART1 at 115200 baud, 8 data bits, no parity, 1 stop bit (8N1) | SHALL | Test |
| PDS-UART-002 | The UART driver shall support transmit of null-terminated strings up to 256 characters | SHALL | Test |
| PDS-UART-003 | The UART driver shall support receive of binary data packets up to 1KB via interrupt-driven reception | SHALL | Test |
| PDS-UART-004 | The UART driver shall detect and report frame errors, parity errors, and overrun errors | SHALL | Test |
| PDS-UART-005 | The UART driver shall complete a 256-byte transmission within 30ms at 115200 baud | SHALL | Analysis |

## 5.4 GPIO Driver (gpio_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-GPIO-001 | The GPIO driver shall configure PA5 as a push-pull output at 2MHz for LED control | SHALL | Test |
| PDS-GPIO-002 | The GPIO driver shall configure PA9 as alternate function push-pull output for USART1 TX | SHALL | Test |
| PDS-GPIO-003 | The GPIO driver shall configure PA10 as alternate function input for USART1 RX | SHALL | Test |
| PDS-GPIO-004 | The GPIO driver shall provide set, reset, and toggle operations for PA5 LED pin | SHALL | Test |

## 5.5 CRC Module (crc_module)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-CRC-001 | The CRC module shall compute CRC-32 (polynomial 0x04C11DB7) over a given memory range using the STM32F303 hardware CRC unit | SHALL | Test |
| PDS-CRC-002 | The CRC module shall reset the hardware CRC unit before each computation | SHALL | Test |
| PDS-CRC-003 | The CRC module shall complete CRC-32 computation over 224KB in less than 100ms | SHALL | Test |
| PDS-CRC-004 | The CRC module shall store the expected CRC-32 value at address 0x08007FFC (last 4 bytes before application) | SHALL | Review |

## 5.6 SysTick Driver (systick_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-TICK-001 | The SysTick driver shall configure the SysTick timer to generate an interrupt every 1ms at 72MHz system clock | SHALL | Test |
| PDS-TICK-002 | The SysTick driver shall provide a HAL_GetTick() equivalent function returning elapsed milliseconds since reset | SHALL | Test |
| PDS-TICK-003 | The SysTick driver shall provide a HAL_Delay() equivalent function with 1ms resolution | SHALL | Test |
| PDS-TICK-004 | The SysTick tick counter shall not overflow within 49 days of continuous operation (uint32_t wraparound) | SHALL | Analysis |

## 5.7 Watchdog Driver (wdg_driver)

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-WDG-001 | The watchdog driver shall configure the IWDG with a timeout of 1 second using the LSI oscillator (~40kHz) | SHALL | Test |
| PDS-WDG-002 | The watchdog driver shall provide a refresh (kick) function that resets the IWDG counter | SHALL | Test |
| PDS-WDG-003 | The watchdog driver shall be started during bootloader initialization and shall not be stopped | SHALL | Review |
| PDS-WDG-004 | The software shall refresh the IWDG at least once every 500ms during normal operation | SHALL | Test |
| PDS-WDG-005 | If the IWDG expires (system hang), the MCU shall perform a system reset automatically | SHALL | Test |

# 6. Minimal Application Requirements

The minimal application is a simple LED blink program whose sole purpose is to confirm that the bootloader successfully transferred execution and the full I-SCEET toolchain produced working code.

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-APP-001 | The application shall toggle the LED on PA5 at a frequency of 1Hz (500ms ON, 500ms OFF) using the SysTick timebase | SHALL | Test |
| PDS-APP-002 | The application shall send the message 'I-SCEET APP RUNNING' via UART once upon startup | SHALL | Test |
| PDS-APP-003 | The application shall refresh the IWDG watchdog every 500ms to prevent system reset during normal operation | SHALL | Test |
| PDS-APP-004 | The application shall run indefinitely without crashing or requiring a reset under normal operating conditions | SHALL | Test |
| PDS-APP-005 | The LED blink pattern shall serve as the visual validation indicator that the full toolchain (M1→M6) has produced correct working code | SHALL | Demonstration |

# 7. Safety Requirements (DAL A)

The following safety requirements apply to the entire software system (bootloader + drivers + application) at DAL A:

| PDS ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| PDS-SYS-001 | The software shall detect any corruption of the application firmware before execution using CRC-32 verification | SHALL | Test |
| PDS-SYS-002 | The software shall never execute corrupted or unverified application code under any circumstances | SHALL | Test |
| PDS-SYS-003 | The software shall use the IWDG watchdog to recover from software hangs — the watchdog shall not be disabled at any point | SHALL | Review + Test |
| PDS-SYS-004 | The software shall be complete and ready for application jump within 500ms of power-on reset | SHALL | Test |
| PDS-SYS-005 | The software shall not use dynamic memory allocation (malloc/free) anywhere in bootloader or drivers | SHALL | Static Analysis |
| PDS-SYS-006 | The software shall not use recursion in any function in bootloader or drivers | SHALL | Static Analysis |
| PDS-SYS-007 | The software shall handle all peripheral initialization failures gracefully — no undefined behavior on hardware fault | SHALL | Test |
| PDS-SYS-008 | All software produced by I-SCEET M3 for this project shall comply with MISRA-C:2012 Mandatory and Required rules | SHALL | Static Analysis |
| PDS-SYS-009 | The software shall achieve full MC/DC structural coverage on all bootloader and driver source files (DAL A requirement) | SHALL | Coverage Analysis |

# 8. Performance Requirements

| PDS ID | Parameter | Requirement | Verification |
| --- | --- | --- | --- |
| PDS-PERF-001 | Boot time | Full boot sequence (init + CRC + jump) ≤ 500ms from power-on | Test — timer measurement |
| PDS-PERF-002 | UART init time | USART1 ready to transmit ≤ 10ms after RCC init | Test — oscilloscope |
| PDS-PERF-003 | CRC computation time | CRC-32 over 224KB ≤ 100ms | Test — timer measurement |
| PDS-PERF-004 | Flash erase time | Single page erase (2KB) ≤ 50ms | Test — timer measurement |
| PDS-PERF-005 | Flash write throughput | Half-word write cycle ≤ 100µs per 16-bit word | Test — timer measurement |
| PDS-PERF-006 | Watchdog refresh period | IWDG refreshed every ≤ 500ms during normal operation | Test — logic analyzer |
| PDS-PERF-007 | LED blink frequency | PA5 toggles at exactly 1Hz ± 5ms measured over 10 cycles | Test — oscilloscope |
| PDS-PERF-008 | RAM usage | Bootloader + drivers total RAM ≤ 8KB (stack + static vars) | Static analysis + map file |
| PDS-PERF-009 | Flash usage | Bootloader + drivers total Flash ≤ 32KB (0x08000200 to 0x08007FFF) | Static analysis + map file |
| PDS-PERF-010 | CPU load | Bootloader main loop CPU usage ≤ 20% at 72MHz during active boot | Analysis |

# 9. External Interface Requirements

## 9.1 UART Interface

| Parameter | Value |
| --- | --- |
| Interface | USART1 — PA9 (TX), PA10 (RX) |
| Baud rate | 115200 baud |
| Data format | 8 data bits, No parity, 1 stop bit (8N1) |
| Voltage | 3.3V logic levels |
| Protocol | Simple ASCII text for debug; binary packet for firmware update |
| Startup message | '\r\n=== I-SCEET BOOTLOADER v1.0 ===\r\n' sent at boot |
| Error message | '[ERROR] CRC FAIL — Safe mode\r\n' sent on CRC failure |
| App ready msg | '[OK] Jumping to application...\r\n' sent before jump |

## 9.2 Firmware Update Protocol

The firmware update interface via UART shall implement the following minimal protocol:

- Host sends command byte: 0xBB to enter update mode

- Bootloader responds: 0xAA (ready) or 0xFF (busy/error)

- Host sends firmware size as uint32_t (little-endian)

- Host sends firmware binary data in 256-byte packets

- Bootloader ACKs each packet: 0xAA (ok) or 0xFF (retry)

- Host sends expected CRC-32 as uint32_t (little-endian)

- Bootloader verifies CRC and responds: 0xAA (valid) or 0xFF (invalid)

- On success: bootloader resets and boots new firmware

## 9.3 LED Interface

| Parameter | Value |
| --- | --- |
| Pin | PA5 — GPIOA Pin 5 |
| Direction | Output — push-pull |
| Speed | 2MHz (low speed sufficient for 1Hz blink) |
| ON state | Logic HIGH (3.3V) — LED illuminated |
| OFF state | Logic LOW (0V) — LED off |
| Blink rate | 1Hz — 500ms ON, 500ms OFF |

# 10. Design Constraints

| Constraint ID | Constraint | Rationale |
| --- | --- | --- |
| CON-001 | Language: C99 only — no C++ or assembly except startup file | DO-178C §7 — MISRA-C:2012 applies to C only |
| CON-002 | No RTOS — bare-metal superloop execution model only | Deterministic timing; simpler MC/DC coverage |
| CON-003 | No dynamic memory allocation (malloc/free/realloc) | DO-178C + MISRA-C Rule 21.3 — non-deterministic |
| CON-004 | No recursion in any function | Unbounded stack depth — stack overflow risk (DAL A) |
| CON-005 | No standard I/O (printf/scanf) in bootloader or drivers | MISRA-C Rule 21.6 — not suitable for safety-critical embedded |
| CON-006 | Bootloader size ≤ 32KB Flash | Leaves 224KB for application firmware |
| CON-007 | Bootloader RAM ≤ 8KB | Leaves 32KB for application runtime |
| CON-008 | All source code generated by M3 shall be compilable with arm-none-eabi-gcc without warnings at -Wall -Wextra | Ensures code quality and portability |
| CON-009 | IWDG watchdog shall be started before CRC verification and shall not be stopped | Safety guardian must be active during all safety-critical operations |
| CON-010 | Application entry point shall be at address 0x08008004 (reset handler) | Fixed by memory map — bootloader jumps to this address |

# 11. Toolchain Validation Criteria

The I-SCEET toolchain is considered validated for this test project when ALL of the following criteria are met:

| # | Validation Criterion | Evidence |
| --- | --- | --- |
| V01 | M1 generates SRD-001 with HLRs covering all PDS-xxx items in this document | SRD-001 + STD-001 traceability PDS→HLR |
| V02 | M2 generates SDDD-001 with LLRs and architecture for all 7 subsystems (BOOT, CLK, FLASH, UART, GPIO, CRC, SYS) | SDDD-001 module decomposition table |
| V03 | M3 generates /src with 7 module subfolders each containing .c, .h, and README.md | /src folder structure + git log |
| V04 | M3-generated code compiles without errors or warnings with arm-none-eabi-gcc -Wall -Wextra | Build log |
| V05 | M4 generates SVCP-LLT-001 with test cases covering all LLRs in SDDD-001 | SVCP-LLT-001 + STD-001 LLR→test coverage |
| V06 | M5 generates SVCP-HLT-001 with test cases covering all HLRs in SRD-001 | SVCP-HLT-001 + STD-001 HLR→test coverage |
| V07 | M6 generates STD-001 with complete bidirectional PDS→HLR→LLR→Code→LLT→HLT traceability | STD-001 + zero orphan report |
| V08 | Firmware flashed to STM32F303 — LED blinks at 1Hz confirming successful boot | Video/photo evidence + UART log |
| V09 | UART outputs 'I-SCEET APP RUNNING' after reset confirming application jump | UART terminal screenshot |
| V10 | MC/DC coverage ≥ 100% on all /src files | Coverage tool report in SVR-001 |

# 12. Approvals

This PDS requires the following approvals before being provided as input to M1:

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Author | Hatem ADOUANI |  |  |
| Project Supervisor | [Supervisor Name] |  |  |
| Systems Engineer | [SE Name] |  |  |
| Quality Assurance | [QA Name] |  |  |